Document:       CINEMU.DOC

Version:        1.2, for use with CINEMU.EXE Version 1.2

Date:           3/17/00

Author:         Zonn Moore

Revision History:

-----------------

Version 1.0, 04/16/97

Initial release.

Version 1.1, 07/31/97

Added mouse support.

Auto speed detection for the different games.

Doubled the number of allowable inputs to 64.

Fixed some grammer.

Printable comments added to .INI files.

Version 1.2, 03/17/00

Added Cinematronics soundboard hardware support.

(I didn't add sounds! There IS NO sound support!)

Overview

--------

This document describes the options availale in the ".INI" files used by the

Cinematronics Emulator CINEMU.

By using a layout similar to that used by "Windows" configuration files I

hope to make the learning curve as small as possible.

The format is simple:

Each section is started with a name in braces.  Ex: [CPU] would start

the section on CPU options.

Each section is followed by any number of options in the format:

option = parameter.  Ex: MemSize = 8k.

Only one Section name or Option per line.

Nothing is case sensitive.

Everything after the characters '#', ';' and '>' is ignored up to the end

of line.  Blank lines are ignored, as are all tabs and spaces between the

keywords.  Any upper/lower case usage, and indentation existing in the

default .INI files is there only to pretty things up.

All comments starting with the '>' character will be printed to the screen.

The order of the Sections, and the order of the parameters in each section

matters little, with the following rules applying:

If more than one section of the same name is given, only the first

one will be used.  And if more than one option of the same name is

given, only the last one will be used.

----------------

There are currently four sections, each containing their own options.

The sections are:

[CPU]

Sets up jumpers and ROM image names needed by the CPU.

[Video]

Sets up jumpers and logic needed to handle the different video modes

available on the Cinematronics hardware.

[Inputs]

Maps a logical name to an input or inputs.  This section maps input

names to an associated input pattern.

[KeyMapping]

Maps keyboard scan codes to the above inputs.  As many keys as desired

may be mapped to a single input name.

Section Details

---------------

[CPU] Options

JMI = [Yes | No]

Default: Yes

This option enables or disables the Jump on Minus jumper that can be found

on the Rev K Cinematronics boards, but can exist as a jumper wire on

earlier revision.

Without this jumper the CPU instruction becomes a JEI (Jump on external

input).  The only game I know of that uses the external input is: Tailgunner.

It's used to read the DAC comparators when determining the current position

of an analog joystick.

This option is game dependent and should not be changed.

----------------

Speed = [Standard | Full]

Default: Standard

This sets the speed of the emulator to either standard, or just for the

heck of it, full speed.

Note: The technique I use to allow for zero flickering of vector emulation

requires the speed of the VGA monitor refresh rate to be faster than the

speed of the Cinematronics hardware's refresh.  This isn't usually a problem

since the Cinematronics uses a 38hz refresh, and I've never heard of even a

CGA monitor refreshing at that rate.  On the other hand when you run the

emulator at full speed it's possible for higher end machines to update at a

*much* faster refresh rate than most VGA screens.  At which point there will

be aliasing, where a previous screen is replaced with a new one before the

previous one was completely drawn.  The result is that FULL speed can cause

much flickering...

----------------

MemSize = [4k | 8k | 16k | 32k]

Default: 8k

Used to set the size of the game images.  This size is the size of all ROM

images added together.  It allows the emulator to account for the different

paging techniques used by the different size games.  It also determines the

extensions used when reading the ROM images.  (See also "RomImages=").

This option is game dependent and should not be changed.

----------------

Inputs = xxxx

Where:

xxxx = A hex number indicating the initial state of the CPU inputs.

Default: FFFF

The default 'open' state of a Cinematronics input is 1 (high).  In most cases

the games expect all the inputs to go high when no button is being pressed.

On some games (Solar Quest, Tailgunner) some of inputs unused by the

control panel are used as extensions to the CPU switches.  This parameter

allows you to effectively place jumpers on the control panel's PCB board

located near the control panel on most games.  Solar Quest uses jumpers

here to set the difficulty level of the game.  Tailgunner uses a jumper

on the control panel to indicate a Digital Joystick is being used in the

place of an Analog Joystick.

A few of the games use the Inputs to set different game options.  These

options are described in the .INI file, using any other setting will probably

cause the emulator to behave erratically.

----------------

Switches = xxxxxxx

Where:

xxxxxxx = Seven binary digits indicating whether a switch is on (1)

or off (0).

All seven switches must always be given.  If a game does not use a particular

switch it should be set to off (0).  This is especially true for Space War,

that uses some of the switch inputs as control panel inputs.

The left most switch setting refers to the top most switch on the CPU card.

This is normally refered to as switch 7.  Switch 1 is the right most setting.

I've made an attempt at defining as many games options as I could find, for

each .INI file.  If they're not defined then I don't know what they are.

----------------

SoundBdPort = xxx

Where:

xxx = A hex number indicating the address of the parallel port used to

drive a Cinematronics soundboard.

Default: 378

This option sets the parallel port address to be used to control the soundboard.

If the proper cable is built, CINEMU will control a Cinematronics soundboard

the same way the real Cinematronics hardware will.  For more information

download the "Cinematronics Soundboard Exorcisor" from

www.zonn.com/Cinematronics/tech.htm

----------------

RomImages = [x:][\][path\]romimage

Standard DOS path that points to the ROM images.  The value 'romimage' is

the root filename only.  The extensions are assumed by the emulator using

the following defaults (based on 'MemSize='):

MemSize=4K

romimage.T7                  ; 2048 byte ROM image

romimage.P7                  ;  "    "    "    "

MemSize=8K

romimage.U7                  ; 2048 byte ROM image

romimage.T7                  ;  "    "    "    "

romimage.R7	                ;  "    "    "    "

romimage.P7                  ;  "    "    "    "

MemSize=16K

romimage.U7                  ; 4096 byte ROM image

romimage.T7                  ;  "    "    "    "

romimage.R7	                ;  "    "    "    "

romimage.P7                  ;  "    "    "    "

MemSize=32K

romimage.U1A                 ; 4096 byte ROM image

romimage.U1B                 ;  "    "    "    "

romimage.U2A	                ;  "    "    "    "

romimage.U2B                 ;  "    "    "    "

romimage.U3A                 ;  "    "    "    "

romimage.U3B	                ;  "    "    "    "

romimage.U4A                 ;  "    "    "    "

romimage.U4B                 ;  "    "    "    "

The variable 'romimage' should be replaced by the name of the ROM

images for your game.

[CPU]

MemSize=16k

Eg: RomImages=C:\CINEROMS\solar

Would indicate to CINEMU that the files:

C:\CINEROMS\solar.u7

C:\CINEROMS\solar.t7

C:\CINEROMS\solar.r7

C:\CINEROMS\solar.p7

should be read.

----------------

Mouse = TAILGUNNER | BOXINGBUGS | SPEEDFREAK | UNUSED

The only three games that supported mouse control were: Tailgunner,

Boxing Bugs, and Speed Freak.  While it might be nice to imagine using a

mouse in single player Star Hawk, the game does not support any type of

analog or optical input.

(See the 'KeyMapping' section on how to setup the Mouse buttons as control

panel inputs.)

----------------

[Video] Options

WinSize = MinX, MinY, MaxX, MaxY

Where:

MinX = The lower X limit of the Cinematronics window to be displayed

in the VGA screen.  (Left edge of screen).

MinY = The lower Y limit of the Cinematronics window to be displayed

in the VGA screen.  (*BOTTOM* edge of screen).

MaxX = The upper X limit of the Cinematronics window to be displayed

in the VGA screen.  (Right edge of screen).

MaxY = The upper Y limit of the Cinematronics window to be displayed

in the VGA screen.  (*UPPER* edge of screen).

The "standard" built into the Cinematronics hardware was a screen whose

starting and ending vectors could start or end any where on a 1024x768 grid,

with the origin (0,0) being in the lower left hand corner of the screen.

Like a graph, as opposed to the standard layout of most bitmapped graphic

screens.

Yet most games start and end somewhere beyond these points.  In some cases

(SpaceWar) this was probably done on purpose to make "flying things" go

halfway off the screen on one side, before wrapping around to halfway on the

screen on the opposite side.

In other cases it seemed to be a matter of how the development monitor, used

to write the game, was "calibrated".  Since messing with the X and Y size

controls on the Vectorbeam monitor can throw the 1024x768 all out of whack.

Negative values are allowed in the MinX and MinY settings.  Defining a larger

window (smaller Min values, and larger Max values), will shrink more of the

Cinematronics screen into the VGA screen.  Reminder: the MaxY value refers to

the *TOP* of the screen.

----------------

Monitor = [BiLevel | 16Level | 64Level | Color]

Indicates what type of monitor the game is expecting.

BiLevel - The standard monitor that allows a Bright and a Normal value

for each vector drawn, and up to 9 levels of intensities for single

points.

16Level - 16 intensity levels are available for vectors and single points.

(Used only by Sundance.)

64Level - 64 intensity levels are available for vectors and single points.

(Used only by Solar Quest.)

Color - 4096 colors available for vectors and single points.

(Used by War of the Worlds and Boxing Bugs.)

This option is game dependent and should not be changed.

----------------

Twinkle = [1 - 10]

Set the "Twinkle" level of stars.  Since the Bi-Level monitor supports

multiple intensities for stars, this sets the level at which point a star

switches from dim to bright.  Space War uses 9, while Star Castle uses 8.

This option is game dependent and should not be changed.

----------------

Rotate = [Yes | No]

A 'Yes' rotates the screen 90 degress clockwise, shrinks the Cinematronics

X axis to fit the VGA's Y axis, and shrinks the Cinematronics Y axis

porportionatly.

Also adds an offset to bring the image to the center of the screen.

----------------

FlipX = [Yes | No]

Make the VGAs X-axis a mirror image of the real image.

This is done AFTER rotation (if requested by setting 'Rotate=Yes').

----------------

FlipY = [Yes | No]

Make the VGAs Y-axis a mirror image of the real image.

This is done AFTER rotation (if requested by setting 'Rotate=Yes').

----------------

Brightness = Red%,Green%,Blue%

Contrast   = Red%,Green%,Blue%

On all 'intensity' based monitor settings (Bi-Level, 16Level and 64Level),

'Brightness=' sets the RGB values for the "Brightest" value displayed, and

'Contrast=' sets the RGB values for the "Dimmest" value displayed (not

including black, which is always black).

The values are given in percentages of maximum.  100% indicating full

intensity, and 0% indicating black.  Don't include the '%' on the parameter

line.

On monitors with more than two intensities, the intermediate values will be

evenly spaced between these two values.

----------------

[Inputs]

This section is used to described inputs to the CPU.

Any character string may be assigned as an input (up to 32 characters), and

up to 64 inputs my be assigned.  The names assigned here will be referenced

later in the [KeyMapping] section.

The bit mapped patterns following the assigned Input name are not as

confusing as they first appear.

First it should be noted that all inputs to the CPU are normally pulled high

and a keypress causes a '1' to change to a '0' value.  That being said, the

values following the assigned Input names, are Hexidecimal representations of

masks that manipulate the state of the CPU's inputs.

The 1st column is the value OR'd with the CPU inputs when a key is pressed.

The 2nd column is the value AND'd with the CPU inputs when a key is pressed.

The 3rd column is the value OR'd with the CPU inputs when a key is released.

The 4th column is the value AND'd with the CPU inputs when a key is released.

Starting at the RIGHT most bit and working our way left:

The lower 16 bits map directly into the 16 inputs of the CPU.  Bit-0 maps to

input 0,... Bit-15 maps to input 15.  Bit-16 through Bit-22 map to the CPU

switches.

Bit 23 maps to the Coin Dropped indicator.  This bit should only be zero'd

by the given masks.  It will be reset by the CPU when it polls this bit

looking for coins.  Because of the auto repeat mode of the PC keyboards,

this bit is usually cleared upon release of a key, if you setup this bit to

work on the pressing of a key, the auto repeat feature will rack up credits

real fast!

The top 8 bits are internally defined by me.  Well for now only bit 24 is,

and is used to indicate the break key has been pressed.  You should leave

the other bits alone.

By clearing more than one bit on a keypress, you can simulated the pressing

of two keys at once.  This is how Barrier is mapped to allow player 1 and

player 2 to use the key pad as the movement control pad.

Also this layout allows the setting and reseting of any repeatable pattern

of bits, which allows it to simulate the 10 to 4 multiplexors used in

Sundance.

None of these inputs have any effect on anything unless they are mapped to

a key in the section [KeyMapping].

----------------

[KeyMapping]

Every option defined under [KeyMapping] must also be defined under the

[Inputs] section.  It is the [KeyMapping] section that maps each of the

previously defined inputs to a key on the keyboard.  The format is simply:

InputName = xx | MB1 | MB2 | MB3

Where 'xx' is the hexidecimal value representing the scan code

assigned to a key on the keyboard.

And MB1, MB2, MB3 are the three buttons of a three button mouse.

MB1 = The left button

MB2 = The middle button

MB3 = The right button

If only a two button mouse is used, the MB3 will be ingored, and MB2

becomes the right button.

The mouse buttons only work if the 'MOUSE=' option, under the '[CPU]'

section is set to something other than 'UNUSED'.

Since there are a zillion different types of keyboards available, and I

wasn't the least bit interested in finding each one and documenting the

scan codes it uses, I slapped together a simple program that displays

the scan code of each key pressed.  It's called SCANCODE.EXE.

Running SCANCODE also lets you see if the combination of keys you want

to use can be pressed simultaneously.  To remove the "ghost" key effect

of matrixed keyboards, most keyboards do not allow unlimited pressing of

keys.  The keys that may be simultaneaously pressed is dependent upon how

each keyboard is internally wired.  Obviously were not talking any kind

of standard here.  You'll also notice the if you press many keys, fast, you

can overun the keyboard's hardware buffer, and a key may appear to be stuck.

This would also appear stuck to the emulator (as it would to any program).

So don't press many keys, fast, and overun the keyboard!  Since tapping the

stuck key unsticks it, this isn't normally a problem.

To assign keys to inputs: Run SCANCODE, write down the scan codes of the

keys you want to use, place a line in the .INI file under the section

[KeyMapping] that maps the scan code to a previously defined input.

A single input may be assigned to multiple keys.

Example:

[Inputs]		; assign CPU input values

; Upon pressing of a key, set Bit-0 to 0. Upon releasing set it back to a '1'

;

;	Press OR'd, Press AND'd, Release OR'd, Release AND'd

Inp0 = 00000000,   FFFFFFFE,    00000001,     FFFFFFFF

[KeyMapping]

; By running SCANCODE we find the <SPACE BAR>'s scancode is 39

Inp0 = 39		; Make the CPU input 0 go, low each time the

; SPACE bar is pressed, and 1 when it's released

Inp0 = MB1             ; Do the same thing when the left mouse button is

; pressed.

----------------

Fini!